Conversion and Processing of Historical Wind Data in Gulf of Mexico

- Introduction

The computer program in this folder is ‘GOM_wind.m’. It was developed to convert and process historical wind data downloaded from NOAA. We collected wind field data which includes wind speed and direction for the region from 98°W to 70°W, 15°N to 35° N, which encloses the entire Gulf of Mexico. The data were downloaded from NOAA website (ftp://polar.ncep.noaa.gov/pub/history/waves/). The wind data were stored in “GRIB” (GRIdded Binary) format , which is a mathematically concise data format commonly used in meteorology to store historical and forecast weather data.

- Input

We downloaded wind data set “WNA” which represents West North Atlantic Ocean. We then extracted wind data from downloaded wind data files, and rewrote it in ASCII format, which can be read with any text editor. We consolidated wind data into one single file for each year, and also created animation files, which enables us to identify significant storm events easily.

- Calculations steps

In Matlab program file ‘GOM_wind.m’, take ‘wna.wind.199907.grb’ for example.

- ‘wna.wind.199907.grb’ from NOAA is transferred as asii data ‘WINDS1.out’ (not attached due to its large size).

- ‘WINDS1.out’ is extracted to WUGRD.mat and WVGRD.mat

- Extract GOM domain from ii and have GOM_WUGRD_file*.mat and GOM_WVGRD_file*.mat.

- Converge iii together to GOMWINDS_file_***_dm8.dat which is in the format of Techplot input file.

- Use iv as Techplot input and export the wind speed and direction animation.

- Step iv is also converged according to the year and export an animation for a whole year from 1999 to 2007 (Sample animation file 199907.avi is attached).

For questions, please contact Dr. Jun Jin at TAMUG. E-mail:jinj@tamug.edu. phone: (409) 740 4878.