1990 CENSUS TRANSPORTATION PLANNING PACKAGE

REGION DEFINITION FILE

Introduction

The region definition file (filename of xxxx.REG, where xxxx= the

CTPP region code) is a control file used in the production

of the CTPP urban element. It contains the name of the region and

its code, defines which counties make up the region, defines the

study area(s) and central business district(s), if any, and their

codes and names. It also defines the type of detailed geography

used (TAZ, tract, or block group), indicates the rates of address

range coverage and block coding necessary for allocated place of

work data to be provided, and lists the codes to identify areas

for which exceptions were made and allocated data were provided,

even though they fell below the coverage and coding rate tolerance

specified.

Syntax of the Region Definition File

A generic example of a region definition file is shown at the

end of this REGION.DOC file. Each line in the region definition

file begins with a key word that describes the information

contained on that line. The key words are:

REGIONCODE

REGIONNAME

REGIONDEF

PACKAGETYPE

STUDYAREACODE

STUDYAREANAME

STUDYAREADEF

CBDCODE

CBDNAME

CBDDEF

CODINGRATE

COVERED

Each key word is followed by a comma, and then a data item. (See

the generic version of a region definition file shown below.)

Most key words are followed by only one data item. For example,

REGIONCODE, REGIONNAME, PACKAGETYPE, STUDYAREACODE, STUDYAREANAME,

CBDCODE, CBDNAME, and CODINGRATE are all followed by only one data

item. The lines which define the region, study area, or CBD, or

designate "covered" places, however, may take more than a single

argument.

The first line of each region definition file contains the

REGIONCODE key word, followed by a comma and the code assigned

to the area. In most cases a four-digit code was assigned, usually

the MSA, CMSA, or PMSA code, or some variant of it. CTPPs for the

states of Connecticut and New Jersey were assigned their respective

state codes (09 and 34) instead of metropolitan area codes. These

2-digit CTPP region codes may have been zero-filled in some cases

and appear as 0009 (CT) or 0034 (NJ).

The REGIONNAME line provides a name for the CTPP for the area.

Oftentimes this is the name of the agency that is the MPO for

the area, or it may be the name of the metropolitan area, county

or state.

The REGIONDEF line lists the codes of the geographic units used to

make up the CTPP region. If the definition won't fit on one line,

multiple lines may be used. The second field on a REGIONDEF line

contains either a two-digit FIPS state code (ss in the generic

example below), or one of the key words MSA or PMSA. The third

field of this line contains a number (n in the generic example)

which indicates how many codes or ranges of codes will follow on

the line. The codes given in the fourth and following fields of

this line designate areas within the state if a two-digit state

code is in field two, or MSA or PMSA codes if one of those key

words is in the second field. All fields, codes, or ranges of

codes are delimited by commas.

Study areas may also be defined (STUDYAREADEF line) using the syntax

described above for the region, although it is more common to define

the study area in smaller units. Thus, the second field on this line

may also contain one of the key words TAZ or TRACT. Where TAZs are

used to define the study area, the third field again contains a

number that indicates how many TAZ codes or ranges of TAZ codes

follow. If the second field contains the key word TRACT, however,

the third field will contain five digits that represent the state

and county codes within which the tracts are located. The fourth

field, in this instance, then contains the number of tract codes

or ranges of tract codes that will follow.

Central Business Districts (CBDs) must be defined as aggregates of

the lowest level of geography used in the region's package. Thus,

for a CBDDEF line the second field must be one of the key words TAZ,

TRACT, or BG. Where the CBD is defined as a collection of TAZs, the

third field on the line will be the number of TAZ codes or ranges

of TAZ codes that follow. Where the CBD is defined as a collection

of census tracts, the third field on the line will contain five

digits that represent the state and county codes within which the

tracts are located. Similarly, if the CBD is being defined by block

group (BG), the third field will contain eleven digits, representing

the state, county, and tract codes within which the block group is

located.

The line beginning with the key word COVERED is used to provide for

exceptions to the rate of address range coverage and block coding

contained on the CODINGRATE line. Generally, the data for any place

which exceeds the stated value in either the extent of address

range coverage or in the rate of block coding will have the benefit

of place of work allocation. That is, all the data will be reported

at the census tract or TAZ level. For places that fall below the

value on both address range coverage and coding rate, no place of

work allocation is utilized. This means that the geographic level

of the data will vary, some will be shown at the tract or TAZ level,

some at the state, county, place level. The COVERED line indicates

the codes of areas (counties, county subdivisions, or places) for

which place of work allocation has been utilized even though the

area falls below the required rate of address range coverage or

block coding as given on the CODINGRATE line of the region

definition file.

Rules for the Region Definition File

Defining the CTPP Region

Region code is 2-4 characters, from 01 to 9999

Region must be defined in terms of whole counties outside New

England or whole MCDs in the six New England states

Region can be defined using MSA or PMSA codes as a sort of

shorthand notation

Defining the Study Area

Do not have to define a study area; e.g., if it's the same as the

region, don't define one

Study area code is 2 characters, 01 to 99

Name will usually be "Study Area". Only when more than one

study area is defined will unique names be needed.

Study area may be defined in terms of the lowest unit of

geography (TAZs, tracts, or BGs)

Study area may be defined using higher level geography (state,

county, state-county-MCD, state-county-MCD-place, state-place).

Study area may be defined using MSA or PMSA codes as shorthand

Defining Central Business Districts (CBDs)

CBD codes are 2 characters, 01-99

CBDs must be defined in terms of the lowest unit of geography

(TAZs, tracts, or BGs)

Name will usually be "CBD". Only when more than one CBD is

defined will unique names be needed.

*************************************************************************

*          GENERIC EXAMPLE OF THE CTPP REGION DEFINITION FILE           *

*                                                                       *

* Note: uppercase characters signify required key words, but they may   *

*       be entered in upper or lower case in the definition file.       *

*************************************************************************

REGIONCODE, xxxx

REGIONNAME,______________________________________________(55

chars. max.)

REGIONDEF, ss, n, ccc, cccmmm, ...

PACKAGETYPE, TAZ (or TRACT, or BG)

STUDYAREACODE, 01 (study area records are optional)

STUDYAREANAME, _______________________________________(40

chars. max.)

STUDYAREADEF, TAZ, n, 1-297, 406873, ...

STUDYAREACODE, 02

STUDYAREANAME, ________________________________________ 40

chars. max.)

STUDYAREADEF, ss, n, ccc, cccmmm, cccmmmpppp, ...

CBDCODE, 01  (CBD records are optional)

CBDNAME, ________________________________________ (40 chars.

max.)

CBDDEF, TAZ, n, 1-446, 700200, ...

CBDCODE, 02

CBDNAME, ________________________________________ (40 chars.

max.)

CBDDEF, TAZ, n, 501-522, 563, ...

CODINGRATE, yy.y

COVERED, ss, n, ccc

COVERED, ssccc, n, mmm

COVERED, ssccc, n, mmmpppp

where:

xxxx   = code for the CTPP region, usually the code for the

largest MSA or PMSA in the area, but may be a

FIPS state code

ss     = a 2-digit FIPS state code

n      = the number of codes or ranges of codes to be listed

ccc    = a 3-digit FIPS county code

mmm    = a 3-digit census MCD code

pppp   = a 4-digit census place code

tttttt = a 6-digit census tract code

yy.y   = the rate of address range coverage or block coding

required in order for allocated place of work data

to be provided. For places with address range coverage

and block coding rates below this percent, the data

will be presented as coded, without the benefit of

further allocation to the census tract and block.