24F97-22

Tracking and Storing Besser Machines parts for Precast Concrete Manufacturing

at

Premier Concrete Products, Inc
38200 Hwy 16

Denham Springs, LA 7070

By

Tripp Gautreaux (24)

Luis Gomez (25)

Timothy Levron (35)

For

IE-4597: Senior Design Project I

Course Instructor:

Dr. Bhaba R. Sarker

Project Advisor:

Dr. Bhaba R. Sarker

On-site Supervisor:

Mr. Cody Colvin, PE

President

Tel: (225) 328-0545; email: ccolvin@premier-concrete.com

December 3, 2024

Abstract

Effective inventory management is essential for optimizing operational efficiency, particularly in environments where parts are frequently stored and retrieved from large storage containers. This project based at Premier Concrete presents the design and implementation of an inventory management system aimed at streamlining the tracking and reordering of parts in storage facilities. These parts are used on the Besser machine which makes precast concrete blocks. The system integrates advanced tracking mechanisms using a software program called Shepherd and an improved layout optimization plan to enhance part accessibility and reduce retrieval time.

The system employs mobile device use of Shepherd and barcode scanning to monitor the movement of parts in and out of storage containers, ensuring real-time inventory updates and minimizing human error. The Shepherd database captures this data, enabling management to have instant access to part availability, location, and historical usage patterns. Shepherd will be programed with a set minimum of each part that should be kept in the containers, and when the current level falls below that value it will trigger a reorder message. This ensures uninterrupted part availability while minimizing overstock and associated storage costs. The part numbers in shepherd correspond to labeled bins in the storage containers going along the shelves. This layout is optimized based on retrieval frequency, size, and usage patterns, ensuring that high-demand parts are positioned for quick access.

In conclusion, the proposed inventory management system not only improves tracking accuracy but also enhances the physical organization of parts within storage containers. By addressing common pain points in inventory handling, the system offers a scalable and efficient solution adaptable to other storage areas in the plant for different machines.

Executive Summary

Our primary objectives were to collect and analyze data on Besser equipment and mold replacement parts to identify usage trends and inefficiencies. With no historical data available, we implemented a logging system for the maintenance crew to document part replacements. This data served as the foundation for our recommendations to optimize storage layout based on ergonomic principles and part usage frequency. Additionally, we plan to integrate Shepherd Maintenance Management Software to streamline operations and improve data accuracy.

The analysis revealed key trends in part replacements. Among equipment components, the Dynapac had the highest replacement frequency, with eight parts replaced, suggesting significant wear or demand. The Vibrator and Car/Crawler followed with moderate replacement frequencies, while other components, such as the Cuber and Mixer, had minimal replacements. For mold components, foundation molds and general molds were the most frequently replaced, with counts exceeding 20 and 30, respectively. Specific parts like 8 RE65 molds and Shoe Screws also exhibited notable replacement activity, while less frequently replaced parts, such as Core Mold and Face Liner, demonstrated greater durability or lower usage. However, limitations in the short data collection period and occasional incomplete logging highlight the need for more comprehensive and consistent data tracking.

Based on these findings, we recommend implementing Shepherd Maintenance Management Software to digitize and streamline maintenance tracking. This will ensure consistent data collection, reduce human error, and provide real-time analytics for more informed decision-making. To further improve operations, we propose designing a storage layout guided by ergonomic principles. Frequently replaced parts, such as Dynapac components and foundation molds, should be stored in easily accessible locations, while less frequently used parts can be grouped in secondary storage areas to enhance workflow efficiency. Finally, extending the data collection period and conducting periodic reviews of Shepherd data will help validate trends, incorporate seasonal variability, and refine both maintenance strategies and storage layouts over time.

By addressing these findings and implementing these recommendations, we aim to enhance operational efficiency, reduce downtime, and create a more ergonomic and data-driven maintenance process that supports long-term productivity.

Table of Contents

Introduction and Background	5

Problem Description and Motivation	5

Management Goal and Objectives	5

Data Collection Performed	6

Data Analysis & Findings	6

Solution Alternatives Identified	9

Recommendations	9

Implementation and Assessment Work Plan	10

Appendices	10

Appendix A – Models and Dimensions of Shelves Inside Containers	10

Appendix B– Excel Logs & Pivot tables with Graphs	13

## Introduction and Background

Premier Concrete Products, founded in 1996, has gained importance in the concrete manufacturing industry, providing masonry, erosion control, precast concrete products, and retaining wall systems. With production facilities in Denham Springs, Louisiana, and a sales office in Houston, Texas, the company serves contractors and retail markets by producing concrete solutions. Their product portfolio includes Keystone retaining wall units, concrete blocks, parking bumpers, barriers, and erosion control products like ShoreFlex and Shoretec systems.

They maintain over 40 molds in their portfolio and produce solutions across various sectors such as infrastructure, landscaping, and construction. The company places a strong emphasis on automation and efficiency to ensure competitive pricing and product quality. In addition to production capacity, their focus on innovation in precast concrete design aligns with growing industry trends toward sustainable and resilient infrastructure. Premier Concrete is currently investing $10 million in expanding its capabilities to include precast concrete pipe production, a strategic move aimed at addressing local infrastructure needs and boosting economic growth. The expansion is expected to create new jobs and further strengthen the company’s role in the regional construction market

## Problem Description and Motivation

Premier currently faces challenges in managing its inventory of spare parts for the maintenance of their hydraulic machines, and a poorly organized storage layout. This lack of organization has led to inefficiencies, such as difficulties in locating necessary parts and overstocking, which increases storage costs. Without a system that tracks every part, the company does not have a reorder point or order needed parts with anticipation, and risks running out of critical parts or holding unnecessary excess inventory. The company does not have records of the part types and amounts they hold in their inventory, and they are all stored without any organization which can cause them to lose parts or order parts they may already have, but cannot find in their storage

## Management Goal and Objectives

The primary goal is to develop a system that optimizes the ordering and tracking of machine parts for Premier Concrete Products, with a focus on improving the efficiency of their spare parts inventory management and reducing associated costs. This system will address the current issues of disorganization and lack of tracking, leading to improved machine uptime and decreased risk of stockouts or overstocking. The objectives are to minimize downtime caused by unavailability of machine parts, streamline the ordering process by reducing lead times, and implement a method for tracking parts removed from storage. Additionally, the system will aim to reduce the total number of parts being stored by focusing on frequently used components, creating a more organized storage layout, and ensuring that all parts are recorded and tracked in real-time. These efforts will contribute to better inventory control, reduce unnecessary expenditures, and improve overall operational efficiency for Premier Concrete Products.

## Data Collection Performed

Given the absence of historical data, we needed to develop a system to track replacement parts currently in use. This was essential to construct a meaningful analysis of the frequency with which various parts are replaced. To address this, we created detailed logs in Excel for the maintenance crew to update after any part of the Besser system was replaced. These logs encompassed all replaced components, including both machine parts and precast molds.

To ensure that the data collection process was straightforward and reliable, we designed the logs to be user-friendly, enabling the crew to quickly input relevant information after performing maintenance tasks. Each entry includes fields for the date of replacement, part description, reason for replacement, and any additional comments about the failure or maintenance process.

Due to the relatively short timeframe for data collection, this process is being updated daily to maximize the accuracy and relevance of the data. While this limited timeframe may not yet allow for long-term trend analysis, it enables us to capture an initial snapshot of the system’s maintenance patterns.

As more data accumulates, we aim to refine our understanding of replacement frequencies and identify any recurring issues. This will serve as a foundation for predictive maintenance strategies and will ultimately support more efficient resource allocation for parts inventory management.

## Data Analysis & Findings

Replacement Frequency of Besser Equipment Parts: The first graph highlights the frequency of replacement for various equipment components within the Besser system over the timeframe of October 30 to November 30. Key findings include:

- Dynapac parts are the most frequently replaced, with a total count of 8 replacements, indicating potentially higher wear-and-tear or operational demand compared to other equipment.

- The Vibrator and Car/Crawler parts follow with moderate replacement frequencies of 3 and 2, respectively.

- Cuber and Mixer parts exhibit minimal replacements, suggesting either lower usage rates or potentially more durable components.

- Some categories remain blank or undefined, which could point to incomplete logging or less relevant equipment categories during this period.

Mold Replacement Frequency: The second graph provides insights into mold replacements from October 30 to December 2, detailing the specific parts replaced and their respective frequencies. Key observations include:

- Foundation molds and Molds (general) are replaced most frequently, with counts exceeding 20 and 30, respectively. This indicates these components experience the highest levels of wear, possibly due to operational intensity or suboptimal maintenance practices.

- 8 RE65 molds and Shoe Screws also exhibit notable replacement counts, indicating areas where material longevity might need evaluation or improvement.

- Several molds, such as the Core Mold, End Liner, and Face Liner, are replaced far less frequently, suggesting lower wear or less frequent use in operations.

- The wide variation in replacement frequencies across molds may reflect differences in operational demand or the durability of the materials used in their construction.

Storage Layout: This was the most difficult to collect data for, but after talking with the maintenance crew we were able to roughly estimate that it took them around 30 minutes each time to locate and retrieve the replacement part with the prior layout. With our new design we aim to drop this time down to 10-15 minutes to locate and retrieve each part. Once the layout is complete on site we will be doing time studies to ensure we improved efficiency.

Labeled diagram of a container wall

## Solution Alternatives Identified

An alternative to the problem with using the Shepherd software to create the inventory management system is to use an Excel sheet. The problem with creating the system in Shepherd is that we have no experience using it for creating a system or being a system user in Shepherd. There is no easy way for us to learn because we cannot install it on our computers and the company has not begun using it at their location. The alternative solution, which uses excel, will be created using an excel sheet that contains all of the part names and numbers with their respective order lead times. There will be a formulated table that will show when inventory levels fall below a set minimum threshold for the number of each part required to be onsite. This alert will also be based on the lead time for each part so that they arrive before the last ones are used up. So far, we have a parts list in excel of all the Besser machine parts, and we have a table with headings that outline important information for tracking and reordering the inventory.

`Using AutoCAD is an alternative to SolidWorks to model our design for the layout and organization of the storage containers. AutoCAD will allow us to make clear 2-dimensional diagrams of our proposed layout. There could be multiple drawings for each side of the 3 containers. Each map can have annotations about where parts should go and information for reordering, such as barcodes and part numbers, so it can be referred to for reordering the correct parts.

## Recommendations

Storage Layout

Storage container layout optimization will be achieved by connecting all containers with doorways to allow for quick access to all ends of the containers. The shelving will be labeled with bin codes to develop a system to locating and restocking the replacement parts. This system will help meet the objective of minimizing the time spent for workers to walk to parts in storage and to make it easier to find specifically which size of certain parts are the correct ones to use for the section of the machine they are repairing. The system will also improve the restocking procedure for replacement parts in the container by making the locations easier to get to and clearly showing where each part belongs to keep the highly detailed system well organized. The new layout and bin labeling has proven to work better than the original, which had an average retrieval time for a part of 30 minutes. Even though only a portion of the parts have been placed back in the new container setup, we have tested walking to, locating, and retrieving some parts to determine that it does take less than 15 minutes. The initial cost of obtaining the containers and setting them up was roughly $5,000, and there are no continuing costs or maintenance associated with the containers. After the storage system gets fully implemented and the workers learn how to use it, they will be able to keep the machine running constantly. The sponsor has reported the machine being down 3-4 hours per day, and the machine will normally make $30,000 daily in product. So the solution should make the company an extra $4000 daily.

Management and Tacking

Shepherd CMMS software will be customized to manage the inventory for The Besser machine, including tracking parts, usage rates, and lead times. The CMMS will be configured to generate alerts based on reorder points and set thresholds for stock levels. This will help meet the objective of keeping management in touch when parts need to be reordered and ordering the correct amount so there is no shortage or overstocking of parts. There is evidence of this shown now with the company having no system in place for tracking and reordering and parts either running out too often or not being used. We will develop Standard Operating Procedures to standardize how staff logs inventory usage and updates the system. Operators will have to add a manual input for when they use a part, and proper capacitation will be used to train them. The storage containers will be set up so the maintenance workers can easily locate and log the parts they are taking, and people restocking the parts will be able to easily identify the correct placement. Training workers on new logging procedures and using new techniques to assist with on-the-job logging will help to meet the objective of keeping up with parts going in and out of the storage containers. These new methods will give the workers an easy and quick way to accurately track which parts they are taking out of storage, since before they did not mark what was being taken, and there was no way of knowing the quantity and type of parts being used. Ordering only the parts necessary will save the company money by not overstocking. Implementing the Shepherd system will cost approximately $2000 per year for the license, and the sponsor estimates that not having unnecessary parts sitting in storage should save approximately $50,000 per year.

## Implementation and Assessment Work Plan

Currently, Premier is implementing the layout that we designed, and they are expected to finalize it in March. When they finalize the implementation of the layout, we will perform time studies on the retrieval of different parts in the storage and perform an ergonomic analysis to minimize the parts' average retrieval time. While Premier implements the layout, we will start familiarizing ourselves with Shepherd’s interface, features, and required variables to make use of those features. After Premier collects enough historical data, we will begin the Shepherd implementation through the tracking logs.

List of tasks:

- Obtain access to Shepherd software and become familiar with it.

- Identify unknown variables to implement Shepherd features.

- Obtain the last of the historical data provided by Premier.

- Implement Shepherd in Premier’s devices.

- Standardize the tracking procedure with operators.

- Perform time studies for part retrieval in the layout.

## Appendices

## Appendix A – Models and Dimensions of Shelves Inside Containers

20 ft Container

1st 40 ft Container

2nd 40 ft Container

## Appendix B– Excel Logs & Pivot tables with Graphs