24F97-28

# DESIGN OF DUNNAGE AND RIGGING METHODS FOR OVERSIZED PRECAST CONCRETE:

# ENSURING STABILITY AND MINIMIZING DAMAGE

By

Mark Magdy HANNA (26)

Ellen HOFFMAN (27)

Carlos PAEZ (43)

For:

IE 4597 Senior Design – Final Report

Course Instructor:

Dr. Bhaba Sarker

Project Advisor:

Dr. Bhaba Sarker

On-site Supervisor:

Chris Ballard

Construction Project Manager

Gainey’s Concrete Company

28021 Coker Vail Rd N, Holden, LA 70744

Telephone: 985-507-1056 | Email: chrisb@gaineysconcrete.com

## Table of Contents

Table of Contents	1

Company Overview:	2

Abstract:	3

Problem Description:	4

Goals and Objectives:	5

Data Collection Performed	5

Data Analysis and Findings	7

Potential Solutions	10

Plywood	11

Hardwood	11

Oriented Strand Board	11

High-Density Polyethylene	11

Steel	11

Implementation and Assessment Work Plan	12

Requirements	12

Constraints	13

Proposed Methodology	14

References	14

Appendix	15

## Company Overview:

Gainey’s Concrete Products, founded in 1981 by Richard Gainey as a residential septic tank installing company is currently a women-owned company by his daughter, Lisa Gainey Roache. The company compromises around 150 employees with the main goal of achieving safety, quality, and innovation in its products. The DOTD and NPCA certified company has grown over the years expanding its portfolio by specializing in producing a diverse range of high-quality precast solutions such as wastewater and stormwater drainage systems, precast vaults, and custom-designed precast components.

In addition to producing precast concrete products, Gainey’s assists clients with support on acquiring necessary permits and navigating interstate shipping logistics for their structures. Not only they provide help on the permits, but they provide professional development education sessions, Lunch n’ Learn, for local companies to help them build camaraderie and professional connections, and to keep the citizens of Baton Rouge safe by having them up to date with the latest local and federal regulations.

For this project, our main focus is to guarantee the safety and on-time delivery of a precast platform system from Holden, LA to East Canaan, CT. Gainey’s intends for us to use different Industrial and Civil Engineering metrics like strengths, statics, quality control, and statistics for the increase of safety and quality of the design for the installation and shipping process of the precast concrete. By applying all those metrics, we will not only try to maximize the safety of the product but also minimize the stress and strain of the precast during the installation and transportation.

## Abstract:

This project addresses the issue of the imbalances and excessive stress put on the concrete chain walls during nationwide transport. These complications can pose significant safety risks and financial burdens, as a falling or broken chain wall may result in lawsuits or necessitate a complete reconstruction of the product. This project analyzes the flaws in the current transportation arrangement and demonstrates alternatives that lower the stress and strain of the chain wall.

This semester, we have completed a SolidWorks model of the original configuration of the chain wall and the dunnage used to dampen forces and vibration during transit. We then transferred this model into Ansys, where we assigned each component its material properties, and then conducted a virtual stress test to determine where the concrete structure is most likely to break. Based on the coloration map in Ansys, the chain walls were most fragile at the top beam.

Next semester, we plan to use this information to focus on designing a new dunnage system that strategically targets the critical points we have identified. We intend on using new materials, such as hardwood, oriented strand board, and high-density polyethylene, as well as new configurations of those materials under the concrete. We will build on the model we have already created to determine multiple options for Gainey’s that meet their budget and safety requirements.

## Problem Description:

Our team will be responsible for improving the padding (referred to throughout this paper as dunnage) and straps that are used to secure a concrete chain wall to an oversized trailer, with a focus on enhancing the safety and quality of the shipment. A chain wall is a basic concrete structure that is used to create stability during construction. It is used to protect against water damage and erosion, or as a simple building-block to connect parts of a building’s foundational elements. When the chain wall is precast, it must be transported to the construction site, which is no small feat. Several challenges and risks could arise during this project. One major issue is the potential risk for unbalanced weight distributions after the precast is installed on the trailer. This imbalance could result from the improper placement or insufficient dunnage supporting the precast. Additionally, we must carefully assess the critical points on the precast to ensure that the dunnage is applied correctly, minimizing the potential risks of cracks. Incorrect use of the rigging methods could lead to the precast falling off the truck during transit. Another risk involves choosing the dunnage material, which if it cannot adequately support the precast or tolerate minor shifts during shipment, could increase the likelihood of damage, instability, and falling.

Additionally, the company has been experiencing issues with the shipment process. After the precast concrete is unloaded at its destination, the steel beams used to support the dunnage are being charged as a shipment on the return trip to the company. Gainey’s seeks a solution to either eliminate the use of these steel beams or a way to reduce the cost associated with their return shipment.

## Goals and Objectives:

The primary goal of the project is to enhance safety via improving the stability of the chain wall, as it is a significant concern for the company when handling the 38,925 lb. load. This involves designing dunnage that ensures secure handling and transport, minimizing the risk of accidents or shifting during transit. Additionally, the project aims to reduce stress and strain on the precast concrete to prevent cracking or damage, achieved by utilizing the best materials that can evenly distribute weight and absorb impact. Lastly, a key objective is to reduce costs as much as possible, selecting cost-effective materials like hardwood and plywood are appropriate, without compromising safety or performance, to ensure long-term savings and project efficiency.

## Data Collection Performed

Our data collection goals were to have a complete understanding of the Chain Wall’s and current dunnage’s structural properties, particularly yield strength and ultimate strength.  We then analyzed these regarding the different kinds of loadings that the concrete will need to withstand in transit – transverse, axial, and torsional.

Gainey’s completes strength testing their concrete in house, so we used their previous data to determine the properties of the concrete we will be working with. The standard “strength” of concrete is the “characteristic strength,” which is 75% of the compression determined by a compressive strength test performed after 28 days of rest. For the specific shipment we are modeling, the compressive strengths at time of shipment are as follows:

Base Pour Cylinders at 7 Days: 5827, 5908 psi

Slab Pour Cylinders at 7 Days: 4434, 4391 psi

Column Pour Cylinders at 7 Days: 5764, 5701 psi

We accomplished our structural analysis via a SolidWorks model of the chain wall and dunnage, which we transferred into Ansys. There we conducted a simulation to determine the weak points of the chain wall with their current dunnage solution. We modeled the chain wall in the simulation after the chain wall that was originally sent from Gainey’s to Connecticut, which has a total of 10 pillars, each of which are 1foot x 1foot. The entire chain wall is 29 feet long, 11.50 feet wide, and 8 feet tall. The concrete is reinforced with rebar up each pillar, and the total structure weighs 38,925 pounds, but Gainey’s produces similarly scaled chain walls that are up to 42,000 pounds.

In Ansys, we simulated the truck hitting a pothole at a speed of 55 miles per hour to test the deformation of the chain wall. To account for the weight of the expanded steel trailer in our simulation, we calculated the force of the impact with a “total weight” value of 68,000 lb using the following equation:

Fimpact=m⋅vt

where m = mass of the trailer (30,844 kg), v = velocity at time of impact (29 m/s), and t = duration of contact with surface (0.1 seconds)

This impact would result in a force of 9.065 MN applied in the negative X, positive Y plane, and then in positive X, negative Y upon landing. (Fig. 1, Fig. 2)

Figure 1. Initial Impact Force on Precast Downwards

Figure 2. Initial Impact Force on Precast Upwards

## Data Analysis and Findings

According to the Precast/Prestressed Concrete Institute, the maximum tensile stress on precast concrete before breaking is determined by applying a factor of safety of 1.5 to the modulus of rupture, shown by the equation below:

fallowable = 7.5fc'1.5=5fc'

Where fallowable is the allowable tensile stress, and f ’c is the concrete compressive strength at time of handling.

Using this equation, we determined that the maximum allowable forces on each of the components of the chain wall are as follows:

fallowableBASE = 7.558271.5=55827=381.7 psi

fallowableSLAB = 7.544341.5=54434=332.9 psifallowableCOLUMN = 7.557641.5=55764=379.6 psi

Therefore, the greatest tensile stress on the chain wall tolerated during transport is 332.9 psi, as this is the lowest allowable value that all parts of the structure can withstand safely.

In our Ansys model, we performed virtual stress tests on the chain wall and dunnage by placing increasing forces along the top and bottom of the entire structure, in order to simulate the typical forces experienced by a trailer on a highway. We determined that the locations most impacted by the applied forces of the road were the top beams and contact points of the columns with the plywood - as shown by the light green areas of the diagram below. (Fig. 3, Fig. 5, Fig. 7) This aligns with our expectations as the beam has less support than the rest of the chain wall, and the columns are absorbing most of the impact.

Our simulation determined that the beam deformed by 4 mm during the initial upwards impact, and then by 4 mm upon landing. These deformations occurred on the plywood, which is concerning, because after multiple impacts, the plywood will greatly deform, causing greater amounts of stress and strain on the concrete. To mitigate this, we need to use a material that does not deform under these large forces, so the chain wall can be protected through long distances that will have many similar impacts. Otherwise, the deformations will increase to a point where the plywood no longer evenly distributes the weight of the chain wall, leading to the stability issues that Gainey’s experienced in their last long-distance transport.

Figure 3. Deformation Caused by Downward Impact

Figure 4. Stress on Stainless Steel Beams (Downward Force)

Figure 5. Deformation Caused by Upward Impact

Figure 6. Stress on Stainless Steel Beams (Upward Force)

Figure 7. Plywood Dunnage Reaching Maximum Deformation

## Potential Solutions

The critical factor in reducing the forces acting on the chain wall is the material of dunnage used underneath. These are some potential materials that we will be testing next semester to determine their dampening efficacy.

#### Plywood

Plywood is the most common dunnage and the least expensive option. However, there could be more beneficial alternatives if the weight increases, though they might be slightly more expensive. For the project we're working on with the company at Gainey’s, after thorough research, Plywood was chosen as the main dunnage material. Nonetheless, we explored other options that might be more durable, able to handle more weight, and be cost-effective.

#### Hardwood

One of the top alternatives we considered was hardwood. Hardwood is denser and stronger than Plywood, allowing it to support heavier loads. While hardwood is rigid, Plywood is more flexible, and Plywood is also significantly cheaper than hardwood. However, hardwood's rigidity comes at the risk of cracking or splitting over time, whereas Plywood’s layered construction makes it less prone to such issues, making it more sustainable in the long run. Despite the higher cost, Hardwood offers the advantage of being able to support heavier weights without the risk of being crushed or overloaded during transport. This durability makes it a reliable option for handling extremely heavy loads, providing greater peace of mind compared to other materials.

#### Oriented Strand Board

In addition to hardwood, other dunnage options were researched. One option is OSB (Oriented Strand Board), which generally costs slightly less than Plywood. While OSB has greater shear strength than traditional Plywood, it is not as strong when it comes to vertical load-bearing capacity.

#### High-Density Polyethylene

Another interesting option is HDPE (High-Density Polyethylene. Depending on the type of HDPE used, the cost can be comparable to or even slightly lower than plywood. HDPE is strong enough to handle heavy loads and is resistant to decay, making it a durable option.

#### Steel

Lastly, Steel dunnage, which is significantly more expensive than Plywood upfront. However, in the long term, it offers the best value due to its extreme strength and reusability. Steel dunnage can support the heaviest precast concrete loads without risking bending or warping, far exceeding the load-bearing capacity of plywood.

## Implementation and Assessment Work Plan

Our next semester deliverables for this project will include two components: a comprehensive written report and a presentation, both of which will cover the project objectives, goals, identified problems, methodology, and the solution we propose. Specifically, we will deliver the following to Gainey’s:

- A set of options to replace or improve the elements currently used in their system, with a particular focus on the dunnage materials.

- A 3D-model design comparing the current system with the proposed updates to help visualize the changes.

  - Individual models for each design idea would be presented

- Data from simulations that calculate the stress and strain forces acting on the precast concrete during shipment to demonstrate the effectiveness of the new system.

- A cost analysis for the different materials options proposed.

- A compilation of all research and references supporting the design ideas.

- A final report documenting the entire project process, from problem identification to solution implementation.

In order to accomplish this, we must keep the following in mind:

### Requirements

- Load Characteristics: The dunnage must support a load exceeding 38,925 lbs without deformation or failure. Suitable materials include hardwood, Plywood, HDPE, and steel due to their strength and durability.

- Material Selection: Materials need to be chosen based on both load-bearing capacity and environmental factors. Hardwood offers high strength but may crack overtime, while HDPE is strong, decay-resistant, and more flexible. Steel offers extreme durability and long-term value despite its higher cost.

- Design for Load Distribution: The design must ensure even weight distribution across multiple support points to prevent damage. Cross-bracing or using multiple dunnage pieces can stabilize the load during transport.

- Compliance and Safety Standards: The dunnage design must meet safety and industry standards, such as OSHA regulations, ensuring safe handling and transport of heavy loads.

- Physical Constraints: The dunnage must be appropriately sized to fit within transportation or storage limits, and it should be easily handled by forklifts or other equipment.

- Cost Efficiency: While steel and HDPE may have higher upfront costs, they are more cost-effective over time due to their durability and ability to handle repeated use without significant wear or risk of failure.

### Constraints

- Safety and Compliance: Ensuring compliance with safety regulations may impose design restrictions, such as material strength requirements or load-bearing guidelines.

- Material Cost: Budget limitations may restrict the use of higher-end materials like steel, requiring a balance between cost and performance.

- Space Limitations: Dunnage size must align with transportation and storage constraints, ensuring it can fit within the available space and be safely handled.

- Handling Equipment Capacity: Dunnage design must be compatible with the weight limits and capabilities of handling equipment. Improper dimensions could pose operational challenges.

- Weight Distribution Complexity: The load must be evenly distributed, which can complicate the design if the load has uneven weight distribution or specific support points.

- Cushioning Trade-offs: While rigid materials like steel provide better structural stability, they may lack cushioning, which could lead to surface damage if not carefully considered.

## Proposed Methodology

Having evaluated the physical properties of the concrete and dunnage, we are now in a position to utilize our findings to address the deficiencies in the current dunnage system. This process will involve an assessment of stability, a comparison of the existing stresses and strains to their respective maximum limits, and the identification of areas where concentrated stresses and strains can be reduced.

To address the design shortcomings we have identified, we will develop additional three-dimensional models of the chain wall loading structure using SolidWorks. These models will allow us to test various dunnage layouts, coupled with different types of dunnage, under both typical and extreme road conditions (such as highway driving and impact from potholes). In parallel, we will conduct structural analyses on the different dunnage materials (such as hardwood and foam) and perform virtual tests to evaluate the performance of various layout and material combinations. The results from these tests will provide a range of options, each corresponding to different cost considerations (including material quantities, pricing, and manpower requirements), along with varying levels of assurance regarding their effectiveness.

Additionally, we will review our Ansys simulation techniques to produce the most comprehensive simulation possible. This will include revising our material properties, the mesh configuration of the chain wall, dunnage, and truck, and further research and implementation of the forces that the system will experience on a typical long-distance trip.

To address the challenge of removing the steel beams, we will explore alternative materials that can offer similar support at a more favorable cost or investigate design configurations that eliminate the need for steel support altogether. Using this approach, along with the methods outlined above, we will present Gainey’s with a range of viable solutions, each at different price points, ensuring the secure delivery of the chain wall in all required scenarios.

## References

Larson, Brian. “Creep and Stress Rupture Properties.” Nondestructive Evaluation Physics : Materials, Iowa State University, www.nde-ed.org/Physics/Materials/Mechanical/Creep.xhtml.

Team, Handyman’s World Editorial. “Plywood vs. Hardwood: Which to Choose?” Handyman’s World, 3 June 2023, handymansworld.net/plywood-vs-hardwood/.

Team, DesignHorizons, et al. “Dunnage in Construction: Materials, Design, Safety, and Applications.” DesignHorizons, DesignHorizons, 14 July 2024, nam04.safelinks.protection.outlook.com/?url=https%3A%2F%2Fdesignhorizons.org%2Fdunnage-in-construction-materials-design-safety-and applications%2F&data=05%7C02%7Cehoffm6%40lsu.edu%7Cd46d1188b7ae4699e18c08dcf2f58c22%7C2d4dad3f50ae47d983a09ae2b1f466f8%7C0%7C0%7C638652383628233285%7CUnknown%7CTWFpbGZsb3d8eyJWIjoiMC4wLjAwMDAiLCJQIjoiV2luMzIiLCJBTiI6Ik1haWwiLCJXVCI6Mn0%3D%7C0%7C%7C%7C&sdata=YfMmL31c7oAgXOma1Jk9LuOYUXga1IHD%2Bf7SwAzdRcQ%3D&reserved=0.

Admin, and Amy Reed. “Softwood Archives.” ThePlywood.Com, 13 Aug. 2024, theplywood.com/category/softwood/.

Brown, Krista. “Calculations for Handling and Shipping Precast Concrete ...” PCI, U.S. Department of Transportation Federal Highway Administration, 2019, www.pci.org/PCI_Docs/GCPCI_Docs/Transportation_Resources/Bridge/PCEF/121535_2019_01_ASPIRE_Panel_Handling_Calcs_-_Brown.pdf.

## Appendix

Figure 8. Plywood Properties

Figure 9. Structural Steel Properties

Figure 10. Concrete Properties

Figure 11. Nylon Properties

Figure 12. Stainless Steel Properties